Peter Gilbey

Gilbert River Dam – 300 000 ML Storage

Estimate

PG 2/11/2010

Based on SunWater Estimate – Aug 2010

| Description | Unit | Quantity | Rate/Amount $ | $Amount(including 30% contingency) |
| --- | --- | --- | --- | --- |
| Direct Costs |  |  |  |  |
| Initial works |  |  |  |  |
| Plant mobilisation, excl RCC and batch plants | Item | 1 | 750 000 | 975 000 |
| Plant demobilisation Excl RCC and batch plants | Item | 1 | 750 000 | 975 000 |
| Erosion and Sediment Control Works | Item | 1 | 240 000 | 312 000 |
| Environmental protection and monitoring | Item | 1 | 200 000 | 260 000 |
| Clear Site and Inundated Area | ha | 3 000 | 1 700 | 6 630 000 |
| Site Access Road | km | 25 | 500 000 | 16 250 000 |
| Road Relocation | Item |  | 6 000 000 | 7 800 000 |
| Concrete Aggregate Preparations | Item | 1 | 1 050 000 | 1 365 000 |
| Haul Roads | km | 10 | 640 000 | 8 320 000 |
| Concrete Plant Operations | Item | 1 | 1 600 000 | 2 080 000 |
| River Care and Diversion | Item | 1 | 2 900 000 | 3 770 000 |
| Foundation Excavation and Preparation | Item | 1 | 6 820 000 | 8 866 000 |
| Foundation Grouting | Item | 1 | 1 100 000 | 1 430 000 |
| RCC Dam | m 3 | 225 000 | 200 | 58 500 000 |
| Spillway | m 3 | 8 400 | 760 | 8 300 000 |
| Outlet works –Concrete structures | m 3 | 9 600 | 1 136 | 14 170 000 |
| Outlet works - Metalwork | tonne | 151 | 13 245 | 2 600 000 |
| Outlet Works – M&E | Item | 2 | 1 470 000 | 3 822 000 |
| Fish Lift | Item | 1 | 7 180 000 | 9 334 000 |
| Fish Lock | Item | 1 | 675 000 | 878 000 |
| Site Services | Item | 1 | 1 500 000 | 1 950 000 |
| Sub Total |  |  |  | 158 587 000 |
| Indirect Costs |  |  |  |  |
| Site Overheads | Item | 1 | 19 000 000 | 24 700 000 |
| Construction Camp | Item | 1 | 10 280 000 | 13 364 000 |
| Profit and offsite overheads | Item | 1 | 15 587 000 | 20 263 000 |
| Sub Total |  |  |  | 58 327 000 |
| Owner Costs |  |  |  |  |
| Concept/Preliminary Design | Item | 1 | 2 000 000 | 2 600 000 |
| Hydrology | Item | 1 | 350 000 | 385 000 |
| Geotech | Item | 1 | 5 000 000 | 6 500 000 |
| Final Design | % Direct Costs | 3% | 4 758 000 | 6 185 000 |
| RCC Mix Design | Item | 1 | 800 000 | 1 040 000 |
| Hydraulic modelling | Item | 1 | 500 000 | 650 000 |
| ECI Costs | Item | 1 | 1 035 000 | 1 345 000 |
| Risk Pool | Item | 1 | 6 500 000 | 8 450 000 |
| EIS | Item | 1 | 7 000 000 | 9 100 000 |
| CHMP etc | Item | 1 | 5 750 000 | 7 475 000 |
| Land Acquisition | Item | 1 | 5 250 000 | 6 825 000 |
| Compensation | Item | 1 | 3 150 000 | 4 095 000 |
| Compliance | Item | 1 | 400 000 | 520 000 |
| Principal’s Insurance | % Direct Costs | 0.75% | 1 169 000 | 1 520 000 |
| LS Levee | % Direct Costs | 0.35% | 545 000 | 708 000 |
| Commissioning | % Direct Costs | 0.5% | 793 000 | 1 031 000 |
| Sub Total |  |  |  | 58 429 000 |
| Total Project Cost |  |  |  | 275 343 000 |